Lesson 2: Electrical Engineering Materials

Overview

### California Content Standard(s) Addressed: - NGSS ETS1.A: Defining and Delimiting Engineering Problems - Crosscutting Concepts: Energy and Matter - Science and Engineering Practices: Developing and Using Models, Analyzing and Interpreting Data, Constructing Explanations and Designing Solutions, Obtaining, Evaluating, and Communicating Information - CCSS.MATH.CONTENT.8.F.A.1: Understand that a function models a relationship between quantities. Graph and analyze how current capacity changes with wire gauge. - CCSS.MATH.CONTENT.8.EE.C.7: Solve equations involving I=V/R to calculate current when given voltage and resistance. - CCSS.MATH.CONTENT.8.SP.A.1: Create scatter plots to analyze bivariate data, such as wire thickness and temperature changes.

Essential Question(s): How do Electrical Engineers choose their materials?

Objective: Students will explore how wires handle electrical current and understand how different factors like wire thickness, insulation, and temperature can impact a wire's capacity to safely carry electricity.

Teacher Background Information + Common Misconceptions: - Common Misconception: Students often think a higher gauge number means a thicker wire. - Clarification Needed: Teacher must emphasize that wire gauge is inverse → lower number = thicker wire. - Understanding Wire Gauges - Teachers should be familiar with how wire thickness, insulation, and heat dissipation are tied to ampacity. - Ampacity is the maximum amount of current (measured in amps) that a wire can carry without overheating or causing damage. If a wire carries too much current, it heats up and can melt insulation, damage components, or even cause a fire. - Thicker wires (lower gauge numbers) can carry more current. Thinner wires (higher gauge numbers) have higher resistance, which creates more heat when current flows. - Insulation is the protective outer layer on a wire that keeps electricity from leaking out or shocking you. - Heat dissipation is how well a wire can release the heat it builds up as current flows. Ampacity assumes the wire can cool itself efficiently. - Common Misconception: Students may think heat causes current to increase. - Clarification Needed: The wire heats up because of resistance to current flow, especially in thinner wires. - Review Ohm’s Law from Lesson 1. - All About Circuits - Ohm's Law - If minimal experience with a multimeter, practice using the multimeter prior to leading this lesson. - Students will be graphing two types of data: - Current vs. Wire Gauge - Temperature vs. Time - Be prepared to model both types of graphs, review axes selection, and talk through interpreting data (trends, anomalies, etc.). - Common Misconception: Students may not understand that relays are electrically operated switches and that fuses are safety components. (needed more for lesson 3) - Clarification Needed: Use real-life analogies to explain. - Imagine you have a light switch across the room, but instead of walking over to flip it, you use a remote control. → The relay acts like that remote; it lets a small current control a larger current in a different part of the circuit. - Pressing a small “Start” button sends a signal to start the powerful engine. → The button is not directly turning the engine; it sends a signal to a relay, which connects the battery to the starter motor.

### Materials - At least three different wire gauges (e.g., 18-gauge, 20-gauge, and 22-gauge wires) - Pre-label gauge types and possibly color-code them - Power supply (low-voltage battery pack or a DC power supply) - Multimeter - Resistors (to limit current and protect the wires) - Lightbulbs - Digital thermometer with a probe or infrared thermometer - Wire strippers - Alligator clip leads (for easy connections)

Student “I can” statements: - I can build a simple circuit and measure current using a multimeter. - I can explain how wire thickness affects the flow of electricity and heat. - I can collect and graph data to show how current and temperature change with wire gauge. - I can use Ohm’s Law (I = V/R) to solve real-world electrical problems. - I can explain why choosing the right wire is important for safety and efficiency in electrical engineering. - I can ask questions and identify areas I want to explore further about electrical circuits and engineering.

Introduction/Engagement/Motivation - Students will build upon the circuits they created in Lesson 1 to understand in depth how electrical engineers determine what materials to use. - Introduce a real-world challenge: “Imagine you’re designing the wiring for an electric vehicle. How do you ensure the wires are safe and efficient?” - Explain that today’s lesson will involve experimenting with different wire gauges to understand their properties and applications.

Sequence of Learning Tasks: - Introduction discussion - Review circuits from the previous lesson and brainstorm new ways to influence or change the brightness of the LED light(s). - Discuss or take notes on necessary vocabulary (printable version at the end of this document) - Circuit - Definition: A circuit is a complete path that electricity can flow through. - Example: A flashlight has a circuit inside. When you turn it on, electricity flows from the battery to the bulb and back. - Parallel circuit - Definition: A type of circuit where electricity flows through more than one path. - Example: In your house, lights are on a parallel circuit so if one light goes out, the others stay on. - Series circuit - Definition: A circuit where electricity flows through only one path. - Example: If you have a string of old holiday lights where one bulb goes out and they all turn off - that’s a series circuit. - Multimeter - Definition: A tool that measures electricity (like voltage, current, or resistance). - Example: You can use a multimeter to check if a battery still has power or to measure how much current is flowing through a circuit. - Wire gauge - Definition: The thickness of a wire. A smaller gauge number means a thicker wire. - Example: An 18-gauge wire is thicker than a 22-gauge wire and can carry more electricity without overheating. - Current - Definition: The flow of electricity through a circuit. - Example: Just like water flows through a hose, current flows through wires and makes things like lights or fans work. - Resistance - Definition: How much a material slows down the flow of electricity. - Example: A thin wire has more resistance than a thick wire. (That’s why thin wires heat up faster.) - Voltage - Definition: The “pressure” that pushes electricity through a circuit. - Example: A 9-volt battery pushes electricity harder than a 1.5-volt battery, so it can power bigger devices. - Ampacity (amps) - Definition: The maximum amount of current (amps) a wire can safely carry without overheating. - Example: If a wire’s ampacity is 5 amps and you try to send 10 amps through it, the wire could overheat or melt. - Insulation - Definition: The protective coating around wires that keeps electricity from leaking out or shocking you. - Example: The plastic around phone charger cables is insulation. This insulation keeps the electricity safely inside the wire. - Ohm’s Law - Definition: A math rule that shows how voltage, current, and resistance are related. The formula is: I = V / R (Current = Voltage ÷ Resistance) - Example: If you have a 9-volt battery and a 3-ohm resistor, the current is 3 amps (9 ÷ 3 = 3). - Teams rebuild simple circuits with more advanced supplies - Provide supplies: - a power source → 2–3 AA Battery Pack (3V–4.5V total) - LED → choose 3mm or 5mm standard LEDs, red is easiest to see - a selection of wires with different gauges (ex: 18-gauge, 20-gauge, and 22-gauge) - Length: approx 12 inches per wire type per group - Colors: Use different colors for easy identification - Resistors (optional) → 220Ω or 330Ω for 3V power supply ### Avoid These for Heat Tests: Power Source; Why Not. Coin Cell Battery; Too weak—won’t generate noticeable heat differences. 9V Battery; Higher voltage but low current capacity—not useful for heating thicker wire. Wall outlet adapters; Can be dangerous if not properly current-limited. - Demonstrate how to use a multimeter. Assess understanding before giving students a multimeter. - Watch the Home Depot video - Read through How to Use a Multimeter - Possible Multimeter Safety Check - Google Form - Measure circuit current - Connect a multimeter in series with the wire to measure the current being drawn by the lightbulb (this will help track the load in the circuit). - Video Example - How to Use a Multimeter - Wire gauges current tests - American Wire Gauge Chart - Start with the thickest wire (18-gauge) and complete the circuit. - Remind students, when comparing metal gauges, the lower the number, the thicker the material. This means a 22 gauge is thicker than a 24 gauge. - Measure the current flowing through the circuit using the multimeter. - Run each circuit for at least 10 seconds to get an accurate reading - Observe how bright the lightbulb gets (brighter light typically means more current is flowing). - Record the Current: Note the current reading from the multimeter. You can also discuss how the wire feels by hand (i.e., whether it feels warm or not). - Repeat for Thinner Wires: Now, switch to thinner wires (e.g., 20-gauge, 22-gauge) and observe and record any changes in current and how the wire feels. - Class discussion - Note - Thicker wires should be able to carry more current with less heat, while thinner wires may heat up faster and carry less current without reaching a dangerous temperature. - What happened when you used a thinner wire? - Did the current change? - Did the lightbulb get dimmer or brighter? - Wire gauges temperature tests - Using a thermometer, students can measure the temperature rise of each wire during the experiment. Record all data in a data table. - Students will run each circuit for 60 seconds to see temp rise clearly. - Have students hold the thermometer to the wire at the start of the test and record temperature. - Demonstrate how to hold the thermometer and record temperature - If using a digital thermometer with a probe: Tape or clip the probe directly to the center of the wire (not near the ends or joints). - If using an IR thermometer: Aim it at the same section of the wire each time, about 2 inches from the middle. - Then have students record the temperature of the wire every 5 seconds for 60 seconds. - Repeat for Thinner Wires: Now, switch to thinner wires (e.g., 20-gauge, 22-gauge) and observe and record temperature changes. ### What You Should See: Wire Gauge; Expected Heating. 18-gauge (thicker); Little or no temp change—low resistance. 20-gauge; Slight temp increase. 22-gauge (thinner); Highest temperature rise—more resistance = more heat. - ⚡SAFETY: - Make sure connections are tight. Loose clips can skew results. - Never let the wire touch itself or short the battery. - Let the wire cool between tests. - Use the same start temp for each wire to compare fairly. - Class discussion - Does the wire heat up more when using thinner wires? - Which wire handles the current without heating up too much? - Why do these tests matter? - Graph data - Students graph the relationship between wire gauge and current capacity (ampacity). For each wire tested, plot current on the y-axis and wire gauge on the x-axis. Ask students to observe and share what they notice. Discuss how ampacity changes as the wire gets thinner. - Students graph the progression of the wire temperature for each wire gauge. For each wire tested, plot temperature on the y-axis and wire gauge on the x-axis. Ask students to observe and share what they notice. Discuss the temperature changes for each wire gauge. - Problem Solving with Ohm’s Law - Ohm's Law is a simple rule that helps us understand how electricity flows through a circuit. - Introduce the equation I=V/R to predict circuit behavior. - Voltage (V): The electrical pressure that pushes the electricity through the circuit. It's like the water pressure in a hose. - Current (I): The flow of electricity. It's like the amount of water flowing through the hose. - Resistance (R): How difficult it is for the electricity to flow. It's like how narrow the hose is. - Provide example problems for students to solve based on their experimental data. - Sample problems below at the end of this lesson.

### Closure/Review/Focus for Next Lesson - Class discussion: - What are you curious about? - What did you learn about how wire thickness impacts current and temperature? - Why is it important for electrical engineers to choose the right wire for their projects? - How might your findings relate to electric vehicles and power systems? - Switch Vehicle connection: - Take out and explore the different wires used in the Switch electric vehicle. - What is the power source? Are different wire gauges being used for different parts of the Switch circuits? What is the importance of each? - Switch Relay connection - Take out and explore the wires used in the switch relay. - What is the power source? Are different wire gauges being used (not just different colors)? What is the importance of this?

### Extensions ### Digital Modeling ### Use Tinkercad to simulate circuits with various wire gauges and analyze performance - Official Guide to TinkerCad Circuits - TinkerCad Circuit Tutorials ### Guest Speaker or Virtual Tour ### Invite an electrical engineer or take a virtual tour of a facility that designs wiring systems for vehicles or renewable energy projects. ### Creative Project ### Design a “Wire Gauge Guide” infographic explaining the properties and applications of different gauges.

### *Additional Lesson Notes: - Temperature data will have some inaccuracies. This lesson is looking at general temperature change between different wire gauges, but will not be exact. - Graphs can be changed depending on how the class collects data. Graphs can also be completed as a class with discussion and questions.

American Wire Gauge Chart - www.electricaltechnology.org/2022/04/american-wire-gauge-awg-chart-wire-size-ampacity-table.html/amp

Lesson 2 Vocabulary

### Circuit: A complete path that electricity flows through.
→ Example: A flashlight has a circuit that sends electricity from the battery to the bulb and back.

### Series Circuit: Electricity flows in one path only.
→ Example: Old holiday lights where one bulb going out makes them all go out.

### Parallel Circuit: Electricity flows through more than one path.
 → Example: Lights in your house—one can go out, and the rest stay on.

### Multimeter: A tool that measures voltage, current, and resistance.
 → Example: Use a multimeter to test a battery or check a circuit’s current.

### Wire Gauge: The thickness of a wire (smaller number = thicker wire).
 → Example: 18-gauge wire is thicker than 22-gauge wire and carries more electricity safely.

### Current (I): The flow of electricity through a wire or circuit.
→ Example: Like water flowing through a hose to spin a waterwheel.

### Resistance (R): How much a material slows down electricity.
 → Example: Thin wires have more resistance—they can get hot faster.

### Voltage (V): The "push" that moves electricity through a circuit.
 → Example: A 9V battery gives more push than a 1.5V battery.

### Ampacity (Amps): The highest amount of current a wire can safely carry.
 → Example: If a wire's ampacity is 5 amps, sending 10 amps through it could melt it.

### Insulation: The protective coating around wires that keeps electricity inside.
 → Example: The rubber on a phone charger keeps it safe to touch.

### Ohm’s Law (I = V ÷ R): Shows how current, voltage, and resistance are connected.
 → Example: A 9V battery with 3 ohms of resistance = 3 amps of current.

Ohm’s Law Practice Problems

### Problem 1: Simple Circuit

You set up a circuit with a 9-volt battery and a resistor of 3 ohms.

- What is the current flowing through the circuit?

### Problem 2: Finding Resistance

In a circuit, a current of 2A flows when a 12-volt battery is connected.

- What is the resistance of the circuit?

------------------------------------------------------------------------------------------------------------------

Problem 1 Solution:

Solution:

The current is 3A

Problem 2 Solution:

Solution:
Rearrange the formula

The resistance is 6Ω