Lesson 5: Graphing CO₂ Emissions, Costs and Fuel Types

Overview

### California Content Standard(s) Addressed: - ESS3.C: Human Impacts on Earth Systems - Crosscutting Concepts: Systems and System Models, Cause and Effect - Science and Engineering Practices: Analyzing and Interpreting Data, Using Mathematics and Computational Thinking - 8.RP.3: Use proportional relationships to solve multistep ratio problems. - 8.EE.5: Graph proportional relationships, interpreting the unit rate as the slope of the graph. - 8.F.1: Understand that a function is a rule that assigns exactly one output to each input.

Essential Question(s): How do different fuel types impact CO₂ emissions, costs, and the environment?

Objective: Students will analyze data on fuel types, CO₂ emissions, and costs to create graphs that compare the environmental and economic impacts of different fuels, fostering an understanding of how these factors influence global climate change and energy choices.

Teacher Background Information + Common Misconceptions: - Be ready to explain how different fuels are measured - Gasoline: per gallon - Natural Gas: per therm - Coal: per ton or per kWh (more complex)

### Materials - Data charts (fuel prices and CO₂ emissions per unit of fuel) - Graph paper or digital graphing tools - Calculators - Chromebooks

Student “I can” statements: - I can explain the relationship between fuel types and CO₂ emissions. - I can calculate the amount of CO₂ produced by burning different fuels. - I can compare the environmental impacts of various fuel types based on CO₂ emissions. - I can analyze fuel cost data and compare it to CO₂ emissions to evaluate economic and environmental impacts. - I can create graphs to visually represent the relationship between fuel type, cost, and CO₂ emissions. - I can discuss how fuel choices influence global climate change and energy consumption.

Introduction/Engagement/Motivation - Show a video about the effects of CO₂ on the environment and the role of fuel types in climate change. - Suggested videos: NASA Climate Change Basics, What is the Greenhouse Effect, Climate Change for Kids. - Class discussion - What do you already know about global warming and climate change? - How do you think fuel types impact the environment? - Use an interactive tool or simulation to explore how CO₂ emissions contribute to global warming. - PhET’s Climate Change module

Sequence of Learning Tasks: - Effects of CO₂ on the environment - Introduce the concept of emissions per unit of fuel. - For every gallon of gasoline burned, about 8.89 kg of CO₂ is released. - Discuss types of fuels and their uses: - Gasoline and Diesel: Common fuels for vehicles; prices fluctuate based on global oil markets. - Natural Gas: Used for heating and electricity; prices change seasonally. - Coal: Primarily for power plants; cost depends on quality and transport. - Propane: Used for heating and cooking; seasonal price variations. - Jet Fuel: Variable due to international oil prices. - Ethanol: Often mixed with gasoline; price linked to crop yields. - Show a table of CO₂ emissions per unit of fuel and fuel prices ### CO₂ Emissions per Unit of Fuel graph at end of lesson. - Have students estimate CO₂ emissions for everyday activities. - Compact Car: A smaller car, like a Toyota Corolla, emits about 0.34 kg of CO₂ per mile. Driving 10 miles would release around 3.4 kg of CO₂. - SUV: A larger SUV, like a Ford Explorer, emits about 0.55 kg of CO₂ per mile. Driving 10 miles would release 5.5 kg of CO₂. - Calculating CO₂ emissions - Step 1: Explain the formula for calculating CO₂ emissions: - Formula: CO₂ = Fuel Used × Emissions per Unit. - Fuel Used: A car drives 10 miles and uses 0.4 gallons of gasoline. Emissions per Unit: Gasoline emits 8.89 kg CO₂ per gallon. Plug the values into the formula CO₂ = 0.4 gallons × 8.89 kg CO₂/gallon Perform the calculation CO₂ = 3.556 kg CO₂ Interpret the result Driving 10 miles in this car emits 3.56 kg of CO₂. - Step 2: Provide data tables (fuel emissions and costs). ### Fuel Prices (Average Estimates) graph at end of lesson. ### CO₂ Emissions per Unit of Fuel graph at end of lesson. - Step 3: In pairs, students - Calculate CO₂ emissions for specific scenarios - e.g., a car using 10 gallons of gasoline - Compare these values with the cost data. - Step 4: Introduce graphing concepts - CO₂ Emissions per Unit of Fuel. - Fuel Prices (Average Estimates). - CO₂ Graphs - Students create two graphs: - Graph 1: CO₂ emissions per unit of fuel. - Graph 2: Fuel prices. - Discuss: - What trends do you notice in the data? - Which fuel type has the lowest emissions? The highest cost? - Class discussion - What did you learn about the relationship between fuel type, cost, and emissions? - How can this information influence personal and societal fuel choices? - Student choice - Each pair or team selects a vehicle (e.g., car, motorcycle, boat, plane) and compares it using the graphs. - Students write out analysis and a brief reflection on their findings.

### Closure/Review/Focus for Next Lesson - Students share vehicle findings with the class. Compare these findings to the Switch EV. - Class discussion around vehicle data. - Does the EV have CO₂ emissions? - Are there any environmental impacts from the EV?

### *Teacher Resources: ### This lesson has led to a lot of class discussion on how different fuel types are measured and how that can be misleading when calculating cost and CO₂ Emissions.

Fuel Prices (average Estimates)

| Fuel Type | Price Range (per unit) |
| --- | --- |
| Gasoline (per gallon) | $3.00 - $5.00 (varies by region) |
| Diesel (per gallon) | $3.50 - $5.50 (higher than gasoline) |
| Natural Gas (per therm) 1 therm of natural gas ≈ 1.14 gallons of gasoline (in terms of energy content) | $1.00 - $3.00 (depending on location and season) |
| Coal (per ton) 1 ton of coal ≈ 170 - 230 gallons of gasoline (energy equivalent, varies by coal type) | $100 - $200 (varies based on region and quality) |
| Propane (per gallon) | $2.50 - $4.00 (varies with location and season) |
| Jet Fuel (per gallon) | $3.00 - $4.50 (for commercial use, can vary widely) |
| Ethanol (per gallon) | $2.50 - $4.00 (depends on region & other requirements) |

CO₂ Emissions per Unit of Fuel

| Fuel Type | CO₂ Produced per Unit |
| --- | --- |
| Gasoline (per gallon) | 8.89 kg CO₂ (about 19.6 lbs) |
| Diesel (per gallon) | 10.16 kg CO₂ (about 22.4 lbs) |
| Natural Gas (per therm) | 5.3 kg CO₂ (about 11.6 lbs) |
| ** Coal (per kWh of electricity) | 0.92 kg CO₂ (about 2 lbs) |
| Propane (per gallon) | 5.5 kg CO₂ (about 12.1 lbs) |
| Jet Fuel (per gallon) | 9.57 kg CO₂ (about 21.1 lbs) |
| Ethanol (per gallon) | 6.89 kg CO₂ (about 15.2 lbs) |

**The EPA uses a standard where 1 gallon of gasoline contains approximately 33.7 kilowatt-hours (kWh) of energy. This is based on the heating value of gasoline.

Take the CO2 emission rate for coal (in kg CO2/kWh) and multiply it by the energy equivalent of a gallon of gasoline (33.7 kWh/gallon). This will give you an estimate of the CO2 emissions associated with producing the same amount of energy from coal as is contained in one gallon of gasoline.

The above is a well-to-wheel value and thus not comparable to the rest of the table.

A metric ton (which is 1000 kg or about 2204.6 pounds) of coal can produce approximately 2,200 kilowatt-hours (kWh) of electricity in a typical power plant.  This will allow some comparison between the price of the energy in relation to the emissions.